Luggage Processing

Patent for Luggage processing

The patent for luggage processing is written by Grube, Hans J. This invention is to find new and improved system for processing passengers during the check-in time at commercial airports and provides security check for both the passengers and their luggage with the minimum number of airport security personnel. The system should be “efficient, unobtrusive and quick manner”

The claim of the patent states that the processing luggage system includes a passenger counters for processing, to verify airline tickets and confirming the identity of the passenger. There would also be equipment for conveying large luggage to be stow onboard the plane from the counter with security checks. There would also be security checks for the passengers and their hand luggage.

There would also be another second conveyor belt that is on the opposite side of the passenger path to the security check point for carry-on or cabin luggage. The second conveyor belt would start ahead of the passenger security check. The luggage would be processed and scan at the same time as passengers are getting verification checks and security checks. When all checks are complete passengers then can pick up their checked hand or cabin luggage onto the plane.

There would also be a ‘weighing system’ provided and installed. The weighing devices is located at the check-in counter on the large luggage conveyor belt. There would also be a weighting system at the security check zone for the carry-on or cabin luggage. The weights of the passengers, carry-on or cabin luggage and the large luggage is weighed and then recorded into a computer or other recording methods. The weight of the passenger is collected at the front door of the plane while waiting for the flight attendance to show you where the seats are located. The weight information is used for landing the plane and also security purposes.

Why they weigh ya without tell ya!

An example of why they weigh you is, the computer can weigh the person and determine that he is 80 kg and his carry-on luggage weigh 10 kg. In the final weight in the plane lost 80 kg meaning that the passenger did not board the plane but his 10 kg luggage did. Security guards would see this as, why didn’t he go on board the plane and his luggage did? How did his luggage go on the plane? What was in his luggage? This is a serious problem which the security team will have to address.

Another example of taking weights is, if the weight of the luggage has decreased or increased significantly since the last weigh-in. Questions like, has the bag been tampered with? These two examples are called a “known security risk”.

http://www.freepatentsonline.com/4137567.html

The Technology being used to assist in the Handling of Luggage

The Baggage Handling System at the airports plays a very important and a crucial part in keeping the travelers happy as the Patent of the Luggage Processing states. There are three main jobs that Handling of Luggage is concern with, these are:

- Move bags from the check-in area to the departure gate

- Move bags from one gate to another during transfers

- Move bags from the arrival gate to the baggage-claim area

To measure the success of a baggage handling system is really simple, can the bags be moved from one point to another as fast as the travelers can? If the bags move slower, customers will get frustrated waiting for the bags, or the bags fail to make the flight on time. Or maybe the bags might even move faster than you in connecting flights and you will miss your bags.

Each airport has its own set of requirements. For example, the time allotted for a bag to make it from for example, the check-in area to the gate. This is determined by how fast a passenger can make the same trip. In some airports, it might only be a short walk to the passenger terminal, while in others, passengers might have to take a train, bus, etc.

Some technology being used for Luggage Processing:

- Destination-coded vehicles (DCVs) - these are unmanned carts propelled by linear induction motors mounted onto tracks that can load and unload bags without stopping.

- Automatic scanners - Scan the labels on the luggage.

- Conveyors - Equipped with junctions and sorting machines automatically route the bags to the gate.

A baggage-handling system is kind of like a road system in a city: The conveyors are like the local roads, the DCV tracks are like the highway and your bag is like the car.

Baggage-handling and road systems share the similar properties such as:

- If a DCV track or a conveyor is blocked or jam the baggage can be routed around the blockage.

- Baggage starts and ends its journey on conveyors just like you when you would drive from home onto the local roads.

- Moving to the DCV track makes the journey longer just like moving from terminal to terminal or gate to gate.

- The DCVs never stop, just as there are no stop lights on a highway.

A DCV highway goes through a tunnel from the check-in area to the passenger terminals.

Picture and Caption from http://travel.howstuffworks.com/baggage-handling1.htm .

However unlike the road system, baggage handling makes all of the decisions on where the bag is going. There are hundreds of computers that keep track of the location of every bag and the schedules of all the planes. The computers systems will control the conveyor junctions and switches in the DCV tracks to make sure each bag ends up exactly where it needs to go.

The Process and the Technology at work

# Check-in

# When you first check in, the agent working at the counter will print out one or more tags to attach to each of the luggage. The tags have information of your flights such as, your destination and transit cities. There is also a bar code with contains ten-digit number. The numbers are unique for your luggage.

# The bag’s first stop after check-in is the automated bar-code scanner. At this station there is an array of bar-code scanners which is arranged at 360 degrees around the conveyor and underneath it as well. This device is able to scan about 90% of the bag that passes through. The rest of the bags are routed to be scanned manually by airport personnel.

Once the baggage-handling system has scanned and read the 10-digit bar-code number, it will now know where your bag is at all times. After this the Conveyors will take each bag to the appropriate destination.

# Going to the Conveyors

There usually hundreds of different conveyors with junctions connecting all of them together. The conveyor system will have to sort all of the bags from the different airlines and send them to the DCVs which are headed to the right terminal. When your bag has been scanned, the baggage-handling system will track the movement of your bag. So when your bag comes to a junction, a machine called a ‘pusher’ will let is go through or push it onto another conveyor leading to another junction.

A BAE pusher slides bags onto another branch of the conveyor system

Picture and Caption from http://travel.howstuffworks.com/baggage-handling3.htm

Using this network of conveyor and junctions your bag can be send to mostly any destination automatically. The last step for the main-terminal conveyor system is the conveyor loading your bag into a passing DCV.

From Conveyor to Destination-coded vehicle

The Destination-coded vehicle or DCV’s job is to move your bag quickly to an off-ramp at the gate. DCVs are deployed at longs distances from the main terminal to the passenger terminals. A DCV is made of metal with wheels on the bottom and a plastic tub on top. The only electronic device it has is a passive radio, a frequency circuit that broadcasts a unique number identifying that particular car. This is similar to the circuit inside anti-shoplifting devices. The DCV can travel up to five times faster than a conveyor almost 20 mph (32 kph).

A conveyor shoots bags into moving DCVs.

Picture and Caption from http://travel.howstuffworks.com/baggage-handling3.htm

How is a DCV Propelled?

DCV are propelled on metal tracks almost like roller coaster. It is propelled by a linear induction motor that is mounted to the tracks. Unlike most electric motors, the linear induction motor has no moving parts. It uses electromagnets to build two magnetic fields, one field is on the track and the other is on the bottom of the DCV. These fields are attracted to each other. The motor moves the magnetic field on the track and pulling the DCV at a high rate of speed. The advantages of this system are, speed, efficiency, durability, precision and lastly manageability. The motors are located about every 50 feet (15 m). Each time a DCV passes through one of the motors, the DCV would receives a boost from the motor allowing it to maintain speed while it coasts to the next motor on the track.

How Does a DCV Load bags?

The plastic Tub on the DCV is mounted on a pivot. This tub most of the time is tilted backward so that the bags don’t fall out. A linkage on the DCV locks the tub in this position. When the DCV comes into a loading area, a bar on the track rises and engages a lever on the DCV. This lever tilts the tub into a flat position to collect the bag. This is done without the DCV stopping so precise timing is needed.

How Does a DCV Unload bags?

Unloading a DCV is a similar process to loading the bags. A mechanism on the track engages a lever on the DCV that causes the tub to tilt downward instead of rising up and dump your bag onto a section of conveyor that runs alongside the track.

In this photo, you can see one DCV in the flat position for loading and another with the tub tilted backward. The blue rail tilts the tub backward.

Picture and Caption from http://travel.howstuffworks.com/baggage-handling4.htm

Photo courtesy BAE Automated Systems, Inc. A DCV unloads a bag onto a conveyer.

Picture and Caption from http://travel.howstuffworks.com/baggage-handling4.htm

Going on the Plane!

There is an off-ramp at every gate in of the terminal. The bags will make their way down a short conveyor to a ‘sorting station’ on the ground of the gate. There will be a monitor at the sorting stations tells the handlers which bags are going where. After the bags are loaded into carts or containers, the bags are brought to a short distance to the plane and then are loaded.

Photo courtesy BAE Automated Systems, Inc. Bags lined up in a sorting station at the gate

Some planes are ‘bulk loaded’ which means the bags are brought up one-by-one on a conveyor and are placed into shelves in the cargo hold. Other planes are ‘container loaded’ which means that special containers are loaded on the ground and then placed into the plane. The sorting station, baggage handlers load the bags onto carts or into special containers that go right into the airplane.

An example of Luggage Handling System

Airport Innovation located in Oslo, Norway has delivered high and low level control systems to many airports for more than a decade. Airport Innovation has provides fully integrated solutions for automated baggage handling which includes Radio Frequency Identification (RFID) and Hold Baggage Screening (HBS).

## Total Baggage Handling and Security Solution from Airport Innovation

Airport Innovation provides the following solutions for its customers:

- Baggage handling systems

- Package sorting systems

- Low level control systems

- High level control systems, Baggage Routing And Information Systems (BRAINS)®

- Different solutions on outline support on computer and PLC systems.

- Screening equipment of baggage through our cooperation partner Bertel O. Steen AS - Smiths-Heimann’s representative in Norway

- RFID

- Biometrics solutions

- 24/7 technical support

Information from, http://www.airport-technology.com/contractors/baggage/airport-innovation/

An example of Baggage Handling Software

Sky Assist is one of the many providers of advance software for airline industry. BagAssist is one of Sky Assist's solutions. It is an application that automates processes for the follow-up of lost and found baggage and is also integrated with SITA WorldTracer™, which is the global standard for the tracing and recovery of lost luggage.

##### Bagassist = Baggage Handling, Scanning and Tracing Software.

The software BagAssist manages the lost and found baggage process. This leads to the increase ground staff productivity and also reduced cost of personnel. BagAssist uses a smart and user-friend GUI which is connected to the internet browser and the centralized database.

BagAssist is currently deployed to clients such as Aéroports de Paris, Air France / KLM, Brussels Airlines, Flightcare, Groundforce, Jet Aviation and Swissport.

##### BagAssist - for your Lost and Found department:

- Fully covering and automating the primary tracing business process, from passenger declaration to delivery of baggage

- Comprehensive user-friendly screens and professional output documents

- Increases staff productivity and reduces training needs

- Increased quality and professionalism towards the passenger and the handled airlines

- Professional approach towards external contractors

- Contains all necessary online information and related triggers

##### BagAutorush - for your Baggage Control department:

- Optimise baggage handling performance to: decrease the number of rushed bags; improve productivity (interfaces supporting automated decision handling); rushed baggage automatically interacts with WorldTracer™

- Manual decisions reduced to nearly zero: staff training done in minutes, no knowledge of WorldTracer™ required; staff low profile requirements

Information from, http://www.airport-technology.com/contractors/consult/sky/

What Issues that have arisen through the use of this Technology

Advantages

- Hardly any of the work done manually

- Decreases stress, confusion, and money

- Luggage can be traced by the printed 10 digit tags

- If done correctly, traveler can expect their bag to arrive on time

- Is quicker and saves time

Disadvantages

- Sometimes there is traffic that occur on conveyors

- If there is any power failure the conveyors, scanners, and DCVs may stop working.

- The machines may malfunction

- Must be maintain

- Some content may be fragile and be broken in the process of moving

- Luggage can still be lost

Social and Ethical Issues

1. Reliability

- The luggage processing system ‘should’ be reliable but sometimes this is not the case

- People put their trust in the system to deliver their luggage on time to much

- If incorrect data is entered about the traveler, their luggage may end up in the wrong place

2. Security

- Sometimes there are random luggage checks. An example of this is in the U.S. only a certain type of lock may be used for locking luggage so they may be opened by airport security in case of inspection

- Only authorized personal may tamper with the luggage

3. Privacy and anonymity

- Since luggage may be opened for inspection, no personal information from the traveler should be recorded

- The traveler should be noted that their luggage may be opened but personnel

4. Control

- Computers have total control over the system

5. Globalization and cultural diversity

- Airports with a better luggage processing systems may attract more travelers and in turn increasing diversity

6. People and machines

- Dependence on machines

- People may begin to lose their jobs to automation